CertMaster Perform Network+

N10-009

  - Lab: Create Network Topologies

In this lab, your task is to create a network design by doing the following:

- Move Marketing's five computers and the switch to the canvas.

- Connect the devices to create a star topology.

Explanation

Complete this lab as follows:

- Move Marketing's five computers and switch to the canvas.

  - In the tools tray, select the End Devices icon.

  - Drag the five computers to the modeler canvas.

  - In the tools tray, select the Switches icon.

  - Drag the Mktg_Switch switch to the canvas.

- Connect the devices to create a star topology.

  - In the tools tray, select the Create Link icon.

  - Select Mktg1 and select the Ethernet port.

  - Select Mktg_Switch and select an open port.

  - Repeat the process for Mktg2-Mktg5.

Lab 1.2: Explore a Single Location in a Lab

You have been hired as a network technician for a small corporate network. Today is your first day on the job, and you need to familiarize yourself with the network. In addition, a new employee will be working in Office 2, and you need to set up their computer.

In this lab, your task is to complete the following from the Hardware view of the Office 2 computer:

Add the monitor to Office 2.

Use the HDMI cable to connect the monitor to the Office 2 computer.

Connect the power cord to the monitor and plug in the monitor.

Add the keyboard and mouse to the Office 2 computer.

Turn on the monitor and the computer.

Explanation

Complete this lab as follows:

- Add a monitor to the Office 2 computer.

  - Under Office 2, select Hardware to go to the workstation.

  - Under Shelf, expand Monitors.

  - Drag the monitor from the Shelf to the bench next to the computer.

- Connect the monitor to the computer.

  - Above the monitor, select Back to switch to the back view of the monitor.

  - Above the computer, select Back to switch to the back view of the computer.

  - Under Shelf, expand Cables.

  - Select HDMI to HDMI Cable.

  - From the Selected Component pane:

    - Drag an HDMI Connector to the HDMI port on the back of the computer.

    - Drag the other HDMI Connector to the HDMI port on the back of the monitor.

- Plug in the monitor.

  - Under Shelf, select AC Power Cable.

  - From the Selected Component pane:

    - Drag the AC Power Connector (Female) to the monitor power port.

    - Drag the AC Power Connector (Male) to the unused power port on the wall plate.

- Add the keyboard and mouse.

  - Under Shelf, expand Input Devices.

  - Drag the Keyboard to an empty USB port on the back of the computer.

  - Drag the Mouse to an empty USB port on the back of the computer.

- Power on the monitor and computer.

  - Above the monitor, select Front to switch to the front view of the monitor.

  - Above the computer, select Front to switch to the front view of the computer.

  - Select the power button on the monitor.

  - Select the power button on the computer.
The computer startup process begins, and you are automatically signed into Windows 11.

  - Right-click Start and then select Settings to explore the operating system simulator's capabilities.

  - From the top menu (left side), select Office 2 to switch back to the Hardware view.

Lab 1.4: Create a SOHO Network

In this lab, you will design a small office/home office (SOHO) network using the network modeler:

Complete the tasks below in the order listed.

- Add the computers to the canvas

  - In the tools tray, select the End Devices icon.

  - Drag all three computers to the modeler canvas.

- Add the switch to the canvas

  - In the tools tray, select the Switches icon.

  - Drag the switch to the modeler canvas.

- Add the router to the canvas

  - In the tools tray, select the Routers icon.

  - Drag the router to the modeler canvas.

- Connect the switch to the router

  - In the tools tray select the Create Link icon.

  - Click on the Router and select Port 0.

  - Click on the switch and select an open port.

- Connect the computers to the switch

  - In the tools tray, make sure the Create Link icon is selected.

  - Click on Home-PC1 and select the Ethernet port.

  - Click on the Switch and select an open port.

  - Click on Home-PC2 and select the Ethernet port.

  - Click on the switch and select an open port.

  - Click on Home-Laptop and select the Ethernet port.

  - Click on the switch and select an open port.

  - Select Create Link to end the link tool.

- Verify that a DHCP address is assigned

  - Right-click Home-PC1 and select Launch Windows.

  - In the system tray, right-click the network icon and select Network and Internet Settings.

  - Select Ethernet and scroll down to the assigned IP address.

  - Answer the Questions.

- Verify connectivity by pinging the gateway

  - Right-click Start and select Terminal (Admin).

  - In the terminal window type ping 192.168.1.1.

  - Press Enter.

Explanation

Complete this lab as follows:

- Add the computers to the canvas

  - In the tools tray, select the End Devices icon.

  - Drag all three computers to the modeler canvas.

- Add the switch to the canvas

  - In the tools tray, select the Switches icon.

  - Drag the switch to the modeler canvas.

- Add the router to the canvas

  - In the tools tray, select the Routers icon.

  - Drag the router to the modeler canvas.

- Connect the switch to the router

  - In the tools tray select the Create Link icon.

  - Click on the Router and select Port 0.

  - Click on the switch and select an open port.

- Connect the computers to the switch

  - In the tools tray, make sure the Create Link icon is selected.

  - Click on Home-PC1 and select the Ethernet port.

  - Click on the Switch and select an open port.

  - Click on Home-PC2 and select the Ethernet port.

  - Click on the switch and select an open port.

  - Click on Home-Laptop and select the Ethernet port.

  - Click on the switch and select an open port.

  - Select Create Link to end the link tool.

- Verify that a DHCP address is assigned

  - Right-click Home-PC1 and select Launch Windows.

  - In the system tray, right-click the network icon and select Network and Internet Settings.

  - Select Ethernet and scroll down to the assigned IP address.

  - Select Questions and answer the questions.

  - Minimize the Lab Questions dialog.

If the network is still not showing a proper connection, restart the Home-PC1 computer. This will force the computer to request an updated IP address from the router.

- Verify connectivity by pinging the gateway

  - Right-click Start and select Terminal (Admin).

  - In the terminal window type ping 192.168.1.1.

  - Press Enter.

2.1.7 Lab: Reconnect to an Ethernet Network

You are a network technician for a small corporate network. The employee in Office 2 reported that they can no longer communicate with other workstations on the local network or the internet. After troubleshooting the problem, you have determined that the built-in network card in the workstation has stopped working.

In this lab, your task is to:

- Select and install a new network card that provides the fastest Ethernet available from the cards you currently have in the office.

- Connect the workstation to the local network using the existing cable.

- After the fix has been implemented, confirm that the workstation has a connection to the local network and the internet.

Explanation

Complete this lab as follows:

- Add the 1000Base TX adapter to the Office 2 computer.

  - Above the computer, select Motherboard to switch to the motherboard view of the computer.

  - Under Shelf, expand Network Adapters.
Identify the network adapter that has the fastest Ethernet speed.

  - Under Shelf, drag the Network adapter, Ethernet 1000BaseTX, PCIe network adapter to a free PCIe slot on the computer.

- Connect the computer to the network.

  - Above the computer, select Back to switch to the back view of the computer.

  - Drag the RJ45 Shielded Connector from the motherboard's NIC to the port of the 1000BaseTX network adapter.

- Verify the connection to the local network and the internet.

  - Above the computer, select Front to switch to the front view of the computer.

  - Select the power button on the computer case.
Wait for the operating system to start.

  - Right-click Start and then select Settings.

  - Select Network & Internet.
The diagram should indicate an active connection.

2.2.9 Lab: Connect to an Ethernet Network

You are a network technician for a small corporate network. The owner of the company has asked you to connect a new workstation to the network while the new employee is in an orientation meeting.

In this lab, your task is to:

- Connect the workstation in Office 1 to the Ethernet local area network using a twisted-pair cable.

- Confirm that the Office 1 workstation is connected to the local network and the internet.

Explanation

Complete this lab as follows:

- Access the back view of the computer in Office 1.

  - Under Office 1, select Hardware.

  - Above the computer, select Back to switch to the back view of the computer.

- Connect the RJ45 cable to the computer and the wall plate.

  - Under Shelf, expand Cables.

  - Select the Cat6a Cable, RJ45 cable.

  - From the Selected Component window:

    - Drag an RJ45 Shielded Connector to the Ethernet port on the computer.

    - Drag the other RJ45 Shielded Connector to the Ethernet port on the wall outlet.

- Test the connection to the internet.

  - On the computer monitor, select Click to view Windows 11 to view the running operating system.

  - Right-click Start and then select Settings.

  - Select Network & internet
The diagram should indicate an active connection.

2.3.6 Lab: Explore Multiple Locations in a Lab

You were recently hired as a network technician for a small corporate network. Your new manager has just asked you to add a network card to a computer, currently in Office 2. You have also been asked to test the UPS, which is providing power to the Networking Closet.

In this lab, your task is to complete the following:

On the Office2 computer:

- For safety, power off the Office 2 computer and unplug the computer from the power source before working with internal components.

- Install the PCIe network adapter in the free PCIe (x1) slot.

- Install a networking cable between the wall plate and the computer.

  - When you make the connection to the wall plate, use the port with the computer symbol embossed on the red box. This is the network port in Office 2.

  - When you make the connection to the computer, use the built-in network adapter on the motherboard or the recently installed network adapter.

- After you install the network adapter, plug in the computer and press the power button to start the computer.

- Using the Settings app, verify that the computer has a connection to the network and internet.

In the Networking closet:

- Test to see if the uninterruptable power supplies are functioning properly in the networking closet.

  - Remove all three power cables from the wall plate. These are for the two rack-mounted UPS systems and a smaller UPS device.

  - Ensure that the power and activity lights for the rack-mounted networking devices are still on and flickering. Also ensure that the monitors are still on.

Explanation

Complete this lab as follows:

- For safety, power off the Office 2 computer and unplug the computer from the power source before working with internal components.

  - Under Office 2, select Hardware to view the hardware for this office.

  - On the computer, select the power button to turn the computer off.

  - Select Back to switch to the back view of the computer.

  - Unplug the AC power cable.

- Install the PCIe network adapter in the Office 2 computer's free PCIe (x1) slot.

  - Above the computer, select Motherboard to switch to the motherboard view of the computer.

  - Under Shelf, expand Network Adapters.

  - Drag the PCIe Network Adapter to the applicable free PCIe (x1) slot on the motherboard.

  - Select Back to switch to the back view of the computer.

  - Reconnect the AC power cable.

- Connect an Ethernet cable to the network card in the computer and to the wall plate.

  - Ensure you are using the Back view of the Office 2 computer.

  - Under Shelf, expand Cables.

  - Select the Networking Cable.

  - From the Selected Component pane:

    - Drag an RJ45 Connector to the built-in port or the port on the PCIe network card.

    - Drag the unconnected RJ45 Connector to the network port on the wall plate (the left port).

- Turn the computer on and test for network and internet connectivity.

  - Above the computer, select Front.

  - Select the power button on the computer.

  - The computer startup process begins and you are automatically logged into Windows.

  - Right-click Start and select Settings to open the Microsoft Settings app.

  - Select Network & Internet.

  - Notice that in the Status pane, the image shown indicates a connection to the network.

- Test to see if the uninterruptable power supply is functioning properly in the networking closet.

  - From the top left navigation tabs, select Floor 1.

  - Under Networking Closet, select Hardware.

  - From the wall outlets, unplug the AC power connectors (Male) by dragging them to the side.

  - Notice that the power and activity lights on the rack-mounted networking devices are still flickering (zoom in if necessary). Also notice that the monitors are still on.

2.3.7 Lab: Connect Network Devices

You're a network technician for a small corporate network. The company recently expanded to the second floor of its building. You've already installed a small networking closet on the second floor, and you've run the necessary cables to the ports on each of the floor's fiber patch panels. Now you need to connect the two networks using fiber optic cables.

In this lab, your task is to connect the switch in the Networking Closet on Floor 1 with the switch in Networking Closet 2 on Floor 2 through the fiber patch panels in each networking closet. Use the following information to identify the necessary connections:

- Connect the appropriate fiber cable to switches through the fiber patch panels.

- For the patch panel on Floor 1:

  - Port 3 is transmit (Tx).

  - Port 4 is receive (Rx).

- For the patch panel on Floor 2:

  - Port 1 is transmit (Tx).

  - Port 2 is receive (Rx).

- Use the color coding on the end of fiber optic cables to identify which end is Tx and which is Rx.

  - Connector A (white or red) is Tx.

  - Connector B (black) is Rx.

- Plug the switch on Floor 2 into a bank 1 critical load outlet on the UPS.

- Verify that the connection was made by checking the internet connection on any Floor 2 computer.

Explanation

While completing this lab, use the following information:

- SC connectors have square connectors that are pushed in to connect.

- LC connectors have both connectors linked together.

Complete this lab as follows:

- Install the SFP Transceiver (LC) in the networking closet on Floor 1.

  - Under Shelf, expand the Adapters.

  - Drag the SFP Transceiver (LC) to the SFP 2 port on the switch.

- Connect the fiber cable to the switch.

  - Under Shelf, expand Cables.

  - Select Cable, Fiber, SC to LC.

  - From the Selected Component pane:

    - Drag Connector, Fiber, Duplex LC Multi-mode, Male to the SFP LC port (plugged into SFP2) on the switch.

    - Drag the Fiber Optic SC Connector (A) to port 3 on the fiber patch panel.

    - Drag the Fiber Optic SC Connector (B) to port 4 on the fiber patch panel.

- Access the networking closet on floor 2.

  - From the top left, select Floor 1.

  - Under Building A, select Floor 2.

  - Under Networking Closet 2, select Hardware.

- Connect the fiber cable to switches on Floor 2.

  - Under Shelf, expand Adapters.

  - Drag SFP Transceiver (LC) to an open SFP port on the switch.

  - Under Shelf, expand Cables.

  - Select Cable, Fiber, SC to LC.

  - From the Selected Component pane:

    - Drag the Connector, Fiber, Duplex LC, Multi-mode, Male to the SFP port.

    - Drag the Fiber Optic SC Connector (A) to port 1 on the fiber patch panel.

    - Drag the Fiber Optic SC Connector (B) to port 2 on the fiber patch panel.

- Plug the switch on Floor 2 into a bank 1 critical load outlet on the UPS.

  - Above the rack, select Back to switch to the back view of the rack.

  - Under Shelf, select AC Power Cable.

  - From the Select Connector pane:

    - Drag AC Power Connector (Female) to the AC port on the back of the switch.

    - Drag the AC Power Connector (Male) to an open bank 1 critical load outlet.

  - Above the rack, select Front to switch to the front view and confirm that the network switch has power.

- Verify that there is an internet connection for any Floor 2 computer.

  - From the top left, select Floor 2.

  - Select any of the computers on Floor 2.

  - Right-click Start and then select Settings.

  - Select Network & Internet.
The image shown should indicate a connection to the internet.

4.1.6 Lab: Explore Packets and Frames

Required Actions and Questions

Isolate traffic with the http.request.method==POST filter

Q1Which layer 2 protocol is used to transfer frames with encapsulated packets?

Your answer:

Correct answer: Ethernet

Q2What is the source address for Ethernet?

Your answer:

Correct answer: 00:00:1c_aa:bb:dd

Q3What is the destination address for the IP Protocol?

Your answer:

Correct answer:62.200.135.84

Q4This packet is using IP version 6.

Your answer:

Correct answer:False

Q5What is the IP header length?

Your answer:

Correct answer:20 bytes

Q6Which encapsulated protocol is listed in the Internet Protocol section?

Your answer:

Correct answer:TCP

Q7What is the TCP destination port?

Your answer:

Correct answer:80

Q8Which TCP flag is set?

Your answer:

Correct answer:Acknowledgement

Q9Under the HTML form, what is the username?

Your answer:

Correct answer:bmoore@corpnet.xyz

Scenario:

In this lab, you will use Wireshark to capture traffic and analyze an entry to see the encapsulation and packet information.

Do the following:

- Use Wireshark to capture packets from the enp2s0 interface (about 5 seconds).

- Apply a display filter to show HTTP POST requests (http.request.method==POST).

- In the packet list pane, select one of the packets.

- Use the information in the packet details pane to answer the questions 1-9.

Enp2s0 Explanation

Complete this lab as follows:

- Begin a Wireshark capture.

  - From the Favorites bar, select Wireshark.

  - Maximize the window for easier viewing.

  - Under Capture, select enp2s0.

  - Select the blue fin to begin a Wireshark capture.

  - Wait about 5 seconds, then select the red square to stop the Wireshark capture.

- Apply a filter for HTTP POST traffic.

  - In the Apply a display filter field, type http.request.method==POST and press Enter to show the HTTP POST requests.

- Select a packet and answer the questions.

  - In the packet list pane, select one of the packets

  - Select Questions

  - In the packet details pane, expand the packet sections to view the packet details.

  - Answer Questions 1 and 9.

4.1.7 Lab: Explore ARP in Wireshark

As a network engineer, it is essential to understand how protocols work. In this lab, you will capture and analyze Address Resolution Protocol (ARP) packets using Wireshark.

Your task is to do the following:

- Use Wireshark to capture packets from the enp2s0 interface (about 5 seconds).

- Apply a display filter to show arp requests (arp).

- In the packet list pane, select a packet where the info starts with Who has.

- Use the information in the packet details pane to answer the questions 1-2.

- Apply a display filter to show ARP requests with the target of 192.168.0.147 (arp.dst.proto_ipv4==192.168.0.147).

- Select a packet indicating where 192.168.0.147 is at.

- Use the information in the packet details pane to answer the questions 3-4.

Task Summary

Required Actions and Questions

Isolate traffic with the arp filter

Q1The Who has ip_address is the same as the ARP Sender IP address

Correct answer: False

Q2Which ARP field stores the Tell ip_address?

Correct answer: Sender IP address

Isolate traffic with the arp.dst.proto_ipv4==192.168.0.147 filter

Q3Which fields contain the same MAC address received for 192.168.0.147?

Correct answer: Ethernet Source and ARP Sender MAC address fields

Q4How many devices responded to the request for 192.168.0.147?

Correct answer: 3

Explanation

Complete this lab as follows:

- Begin a Wireshark capture.

  - From the Favorites bar, select Wireshark.

  - Maximize the window for easier viewing.

  - Under Capture, select enp2s0.

  - Select the blue fin to begin a Wireshark capture.

  - Wait about 5 seconds,then select the red square to stop the Wireshark capture.

- Apply the arp filter.

  - In the Apply a display filter field, type arp and press Enter.

  - In the packet list pane, select a packet where the info starts with Who has.

  - In the packet details pane, expand Address Resolution Protocol.

  - Select Questions, then answer Questions 1 and 2.

  - Minimize the Lab Questions dialog.

- Apply the arp.dst.proto_ipv4==192.168.0.147 filter.

  - In the Apply a display filter field, type arp.dst.proto_ipv4==192.168.0.147 and press Enter.

  - In the packet list pane, select a packet indicating where 192.168.0.47 is at.

  - In the packet details pane, expand Ethernet and Address Resolution Protocol.

  - Select Questions, then answer Questions 3 and 4.

4.2.9 Lab: Configure IP Addresses

You work as the IT administrator for a small corporate network. You need to configure the workstation in the executive office so that it can connect to the local network and the internet. The workstation has two network interface cards (named Ethernet and Ethernet 2). Having two network cards allows the workstation to connect to the local network (as shown in the exhibits) and another small network, which is not yet built.

In this lab, your task is to:

- Configure IP version 4 TCP/IP using the settings specified in the table below (for both network cards).

- From the Exec computer, ping the preferred DNS server assigned to the Ethernet NIC to verify that it can communicate successfully.

| TCP/IP Setting or Info | Ethernet | Ethernet 2 |
| --- | --- | --- |
| Subnet | 192.168.0.0/24 | 10.0.0.0/16 |
| IP address | Use the last valid address on the subnet. | Use the last valid address on the subnet. |
| Subnet mask | Use the default mask that matches the Class C subnet. | Use the default mask that matches the Class B subnet. |
| Default gateway | Choose the appropriate address for the router as shown in the exhibits. | Do not configure a default gateway value. |
| Preferred DNS server address | Use the address of an external DNS server as shown in the exhibits. | Do not configure a DNS value. |
| Alternate DNS server address | Use the address of an external DNS server as shown in the exhibits. | Do not configure a DNS value. |

Task Summary

Required Actions

Configure settings for Local Area Connection Show Details

Configure settings for Local Area Connection 2 Show Details

Successfully ping the preferred DNS server

Explanation

Complete this lab as follows:

- Access the properties for the NIC named Ethernet.

  - Right-click Start and then select Settings.

  - Select Network & internet.

  - From the right pane, select Ethernet.

- Configure the IP version 4 TCP/IP settings for the Ethernet NIC.

  - For IP assignment, select Edit.

  - Configure the IPv4 settings as follows:

    - IP address: 192.168.0.254

    - Subnet mask: 255.255.255.0

    - Default gateway: 192.168.0.5

    - Preferred DNS: 163.128.78.93

    - Alternate DNS: 163.128.80.93

  - Select Save.

- Configure the IP version 4 TCP/IP settings for the Ethernet 2 NIC.

  - From the left pane of the Settings app, select Network & internet.

  - From the right pane, select Advanced network settings.

  - Select More network adapter options.

  - From the Network Connections window, right-click Ethernet 2 and then select Properties.

  - Select Internet Protocol Version 4 (TCP/IPv4).

  - Select Properties.

  - Select Use the following IP address.

  - Configure the Internet Protocol information as follows:

    - IP address: 10.0.255.254

    - Subnet mask: 255.255.0.0

    - Default gateway: None

    - Preferred DNS server: None

  - Select OK.

  - Select Close.

- Ping the preferred DNS server assigned to the Ethernet NIC.

  - Right-click Start and select Terminal (Admin).

  - From the PowerShell prompt, type one of the following:

    - ping 163.128.78.93

    - ping 163.128.80.93

  - Press Enter.

5.1.9 Lab: Install an Enterprise Router

Your company has experienced significant growth and recently purchased additional space in Building A. Your team of network engineers has worked hard to complete the infrastructure for Building A computers and servers. The final area to be completed is in the basement. You will install the CorpNet router as shown in the network diagram (see the exhibit) and make the necessary connections. The fiber patch panel is already connected to other floors.

To install the router and complete the connectivity, perform the following tasks.

- Add the router to the rack in the open spot near the top.

- Install the necessary SFP Transceiver adapters.

  - Insert an SFP Transceiver (RJ45) in the WAN port on the router.

  - Insert an SFP Transceiver (LC) in the LAN 1-4 ports on the router.

- Use the AC power cable with C14 end to connect the router to a critical load bank outlet on one of the UPSs.

- Select the necessary fiber cable and connect the LAN ports on the router to the appropriate port on the fiber patch panel.

  - To properly connect to the fiber patch panel, use the color coding on the end of the fiber optic cables to identify which end is Tx and which is Rx.

    - Connector A (white or red) is Tx.

    - Connector B (black) is Rx.

  - The fiber patch panel has the odd ports set as Tx and the Even numbered ports set as Rx.

  - Make the following connections:

    - LAN 1 port on the router to ports 1 and 2 on the fiber patch panel

    - LAN 2 port on the router to ports 3 and 4 on the fiber patch panel

    - LAN 3 port on the router to ports 5 and 6 on the fiber patch panel

    - LAN 4 port on the router to ports 7 and 8 on the fiber patch panel

When connecting fiber optic cables, connect the transmit (Tx) port on one device to the receive (Rx) port on the other. The fiber cables in this lab have already had their Rx and Tx connectors switched as necessary. You can view component information in the Details window on the Specification tab.

- Connect the WAN port on the router to a port on the pfSense device at the top of the rack using a Cat6a cable.

Explanation

Complete this lab as follows:

- Add the router to the empty slot near the top of the rack.

  - Under Shelf, expand Routers.

  - Drag the Router to the first open space at the top of the rack in the Workspace.

- Insert an SFP Transceiver (RJ45) into the WAN port on the router.

  - Under Shelf, expand Adapters.

  - Drag the SFP Transceiver (RJ45) to the WAN port on the router.

- Insert an SFP Transceiver (LC) into the LAN 1-4 ports.

  - Under Shelf, expand Adapters.

  - Drag an SFP Transceiver (LC) to each of the LAN 1-4 ports on the router.

- Use the AC power cable with C14 end to connect the router to a critical load bank outlet on one of the UPSs.

  - Above the rack, select Back to switch to the back view of the rack.

  - Under Shelf, expand Cables.

  - Select the AC Power Cable with C14 end.

  - From the Selected Component pane, drag the AC Power Connector to the router and the AC Power Connector C14 to a critical load bank outlet on a UPS.

- Connect the LAN 1 port on the router to ports 1 and 2 on the fiber patch panel using an SC to LC fiber cable.

  - Above the rack, select Front to switch to the front view of the rack.

  - Select the SC to LC fiber cable.

  - From the Selected Component pane:

    - Drag the Duplex LC Fiber Connector to the LAN 1 port on the router.

    - Drag the Fiber Optic SC Connector (A) to port 1 on the fiber patch panel.

    - Drag the Fiber Optic SC Connector (B) to port 2 on the fiber patch panel.

- Connect the LAN 2 port on the router to ports 3 and 4 on the fiber patch panel using an SC to LC fiber cable.

  - Under Cables on the Shelf, select the SC to LC fiber cable.

  - From the Selected Component pane:

    - Drag the Duplex LC Fiber Connector to the LAN 2 port on the router.

    - Drag the Fiber Optic SC Connector (A) to port 3 on the fiber patch panel.

    - Drag the Fiber Optic SC Connector (B) to port 4 on the fiber patch panel.

- Connect the LAN 3 port on the router to ports 5 and 6 on the fiber patch panel using an SC to LC fiber cable.

  - Under Cables on the Shelf, select the SC to LC fiber cable.

  - From the Selected Component pane:

    - Drag the Duplex LC Fiber Connector to the LAN 3 port on the router.

    - Drag the Fiber Optic SC Connector (A) to port 5 on the fiber patch panel.

    - Drag the Fiber Optic SC Connector (B) to port 6 on the fiber patch panel.

- Connect the LAN 4 port on the router to ports 7 and 8 on the fiber patch panel using an SC to LC fiber cable.

  - Under Cables on the Shelf, select the SC to LC fiber cable.

  - From the Selected Component pane:

    - Drag the Duplex LC Fiber Connector to the LAN 4 port on the router.

    - Drag the Fiber Optic SC Connector (A) to port 7 on the fiber patch panel.

    - Drag the Fiber Optic SC Connector (B) to port 8 on the fiber patch panel.

- Connect the WAN port on the router to a port on the pfSense device at the top of the rack.

  - Under Cables on the Shelf, select the Cat6a Cable.

  - From the Selected Component pane, drag one of the RJ45 Shielded Connectors to the WAN port on the router and the other to a port on the pfSense device above the router.

You are the IT security administrator for a small corporate network. You need to increase the Networking Closet's security by implementing a CCTV system with IP cameras. As part of this task, you need to separate the CCTV data traffic on the network using a separate VLAN on the switch.

The patch panel connections for the Networking Closet, Lobby, and IT Administration office are installed and ready for use (ports 18-20). A DHCP server is already configured to provide the IP cameras and the laptop in the IT Administration office with the correct TCP/IP settings (port 21). For an easier implementation, create the logical VLAN first and then establish the physical connections for the IP cameras and the laptop.

In this lab, your task is to:

- Access the switch management console from ITAdmin using the following credentials:

  - Address: http://192.168.0.2

  - Username: ITSwitchAdmin (case-sensitive)

  - Password: Admin$only (case-sensitive)

- Create and configure a VLAN on the switch as follows:

  - VLAN ID: 2

  - VLAN name: IPCameras

  - Configure ports GE18, GE19, GE20, GE21 as untagged.

  - Port 18 is connected to the network jack next to the laptop in the IT Administration office.

  - Port 19 is connected to the camera mount in the Lobby.

  - Port 20 is connected to the camera mount in the Networking Closet.

  - Port 21 is connected to a DHCP server that provides IP addresses to the camera and the laptop.

- In the Lobby and Networking Closet, perform the following:

  - Connect a Cat 6a cable to the RJ-45 ports on the IP camera and the IP camera wall plate.

  - Mount the IP camera on the wall plate.

- In the Networking Closet, connect the DHCP server to the VLAN using a Cat 6a cable from switch port 21 to patch panel port 21 on the rack.

- In the IT Administration office, connect a Cat 6a cable to the laptop's network port and the open port on the wall plate.

- On IT-Laptop5, verify the VLAN configuration and IP camera installation as follows:

  - Select Start > IP Cameras.

  - Verify that the program detects the IP cameras on the VLAN 2 network.

Explanation

Complete this lab as follows:

- Log in to the Cisco switch.

    - In the Username field for the Cisco switch, enter ITSwitchAdmin (case-sensitive).

    - In the Password field, enter Admin$only (case-sensitive).

    - Select Log In.

- Create the IPCameras VLAN.

    - From the Getting Started pane (right), under Initial Setup, select Create VLAN.

    - Select Add.

    - For VLAN ID, enter 2.

    - For VLAN Name, enter IPCameras.

    - Select Apply.

    - Select Close.

- Configure the IPCameras VLAN ports.

    - From the left pane, under VLAN Management, select Port to VLAN.

    - Using the VLAN ID equals to drop-down menu, select 2.

    - Select Go.

    - For ports GE18 through GE21, use the drop-down menus to select Untagged.

    - Select Apply.

- Connect the IP camera in the lobby to the VLAN and mount the IP cameras.

    - From the top left, select Floor 1.

    - Under Lobby, select Hardware.

    - Under Shelf, expand CCTV Cameras.

    - Drag the IP Camera (Lobby) to the workspace.

    - Under Workspace, for the IP camera, select Back to switch to the back view of the IP camera.

    - Under Shelf, expand Cables and then select the Cat6a Cable, RJ45 cable.

    - From the Selected Component pane:

      - Drag an RJ45 Connector to the RJ-45 port on the IP camera wall mount plate.

      - Drag the unconnected RJ45 Connector to the RJ-45 port on the back of the IP camera.

    - Drag the IP camera to the IP camera wall plate.

- Connect the IP camera in the Networking Closet to the VLAN and mount the IP cameras.

    - From the top left, select Floor 1.

    - Under Networking Closet, select Hardware.

    - Under Shelf, expand CCTV Cameras.

    - Drag the IP Camera (Networking Closet) to the workspace.

    - Under Workspace for the IP camera, select Back to switch to the back view of the IP camera.

    - Under Shelf, expand Cables and then select the Cat6a Cable, RJ45 cable.

    - From the Selected Component pane:

      - Drag an RJ45 Connector to the RJ-45 port on the IP camera mount wall plate.

      - Drag the unconnected RJ45 cable to the RJ-45 port on the back of the IP camera.

    - Drag the IP camera to the IP camera wall plate to mount the IP camera.

- Connect the DHCP server and laptop to the VLAN.

    - From the Networking Closet, under Shelf, select Cat6a Cable, RJ45.

    - From the Selected Component pane:

      - Drag an RJ45 Connector to port 21 on the switch.

      - Drag the unconnected RJ45 Connector to port 21 on the patch panel.

- Connect IT-Laptop5 to the VLAN.

    - From the top menu, select Floor 1.

    - Under IT Administration, select Hardware.

    - Above IT-Laptop5, select Back to switch to the back view of the laptop.

    - Under Shelf, select Cat6a Cable, RJ45.

    - From the Selected Component pane:

      - Drag an RJ45 Connector to the RJ-45 port on the laptop.

      - Drag the unconnected RJ45 Connector to the open RJ-45 port on the wall plate.

To verify that all components are connected, you can change the location to the Network Closet hardware view. You should see green link/activity lights on ports 18 - 21 of the switch.

- Launch the IP camera monitoring software.

    - Under the laptop's workspace, select Front.

    - On the IT-Laptop5, select Click to view Windows 10.

    - From the taskbar, select Start.

    - Select IP Cameras.

    - Verify that both cameras are detected on the network.

5.6.10 Lab: Configure Trunking

As a network administrator, you are setting up a new switch and need to configure trunking. You need to secure access to your switch, which is still configured with the default settings.

In this lab, your task is to complete the following:

- From Google Chrome, access the switch console using the following:

  - Site: 192.168.0.2

  - Username: cisco

  - Password: cisco

- Examine the default settings of all your ports.

  - Answer Question 1.

- Set ports GE1 – GE26 to Access Mode.

- Set ports GE27 and GE28 to a port VLAN ID (PVID) of 2.

- Add VLANs 22, 44, and 67 to ports GE27 and GE28.

- Save the changes to the switch's startup configuration file.

Q1What is the default Interface VLAN mode?

Correct answer: Trunk

Set ports GE1 - GE26 to Access Mode Show Details

Set ports GE27 and GE28 PVID to 2 Show Details

Add VLANs 22, 44, and 67 to ports 27 & 28 Show Details

Save and apply your changes

Explanation

Complete this lab as follows:

- Log in to the CISCO switch.

    - Click the Start button, then select Google Chrome.

    - In the URL field, enter 192.168.0.2 and press Enter.

    - Maximize the window for better viewing.

    - In the Username and Password fields, enter cisco (the password is case sensitive).

    - Select Log In.

- Examine the switch port defaults.

    - From the left navigation bar, expand and select VLAN Management > Interface Settings.

    - Using the interface shown in the right pane, examine the settings for all ports.

    - For a detailed view of a single port, you can select Edit.

    - From the upper right, select Questions.

    - Answer Question 1.

    - Minimize the Lab Questions dialog.

- Set ports GE1 through GE26 to Access Mode.

    - From the Interface Settings pane, select GE1.

    - Select Edit.

    - Maximize the window for better viewing.

    - For Interface VLAN Mode, select Access.

    - Select Apply and then select Close.

    - With GE1 still selected, click Copy Settings.

    - In the to field, type 2-26 and then select Apply.
Notice that under the Interface VLAN Mode column, ports GE1-GE26 are now set to Access.

- Set the port VLAN ID (PVID) for ports GE27-GE28 to the value of 2.

    - Select the desired port and then select Edit.

    - For the Administrative PVID, enter 2.

    - Select Apply and then Close.

    - Repeat steps 4a - 4c for the second port.

- Add VLANs 22, 44, and 67 to ports GE27 and GE28.

    - From the left pane, under VLAN Management, select Port VLAN Membership.

    - Select port GE27 and then select Join VLAN.

    - From the new window, hold down the Shift key and select VLANs 22, 44, and 67; then select the > button to assign the VLANs.

    - Select Apply and then select Close.

    - Repeat steps 5b - 5d for port GE28.

- Save the changes to the switch's startup configuration file.

    - From the top of the switch window, select Save.

    - For Source File Name, make sure Running configuration is selected.

    - For Destination File Name, make sure Startup configuration is selected.

    - Select Apply.

    - Select OK.

    - Select Done.

5.6.11 Lab: Configure Switch IP Settings – CLI

You are in the process of configuring the Branch1 switch.

In this lab, your task is to:

- Find the IP address assigned to the FastEthernet0/0 interface on router SFO.

  - Use the show cdp neighbors detail command.

  - Answer the question.

- Configure the switch with the following parameters:

  - Interface: vlan1

  - IP address: 192.168.11.250

  - Subnet mask: 255.255.255.0

- Configure the switch to use the FastEthernet0/0 interface on the SFO router as the default gateway.

- Save your changes to the startup-config file.

CDP Cisco Discovery Protocol Guide with Examples

Q1What is the IP address assigned to the FastEthernet0/0 interface on the SFO router?

Correct answer: 192.168.11.254

Explanation

Complete this lab as follows:

- Find the IP address assigned to the FastEthernet0/0 interface on the SFO router.

  - Select the Branch1 switch.

  - From the Terminal, press Enter to get started.

  - Type enable and press Enter to change to the EXEC or Global Configuration mode.

  - Type show cdp neighbors detail and press Enter.

  - Find the IP address for the SFO router.

  - Answer the question.

  - Move the question dialog to the side and keep working.

- Configure the IP address and subnet mask for the Branch1 switch.

  - At the Branch1# prompt, type config t and press Enter.

  - At the Branch1(config)# prompt, type interface vlan1 and press Enter.

  - At the Branch1(config-if)# prompt, type ip address 192.168.11.250 255.255.255.0 and press Enter.

  - At the Branch1(config-if)# prompt, type exit and press Enter.

- Configure the switch to use the FastEthernet0/0 interface on the SFO router as the default gateway.

  - At the Branch1(config)# prompt, type ip default-gateway (routers_IP_address) and press Enter.

  - At the Branch1(config)# prompt, type exit and press Enter.

- Save your changes to the startup-config file.

  - At the Branch1# prompt, type copy run start and press Enter.

  - Press Enter to begin building the configuration.

  - When you see OK, press Enter.